// Mega2560 sketch — Serial3 protocol receiver

#include <Servo.h>

Servo myservo;

Servo right;

Servo left;

String buf = "";

int left_percent = 50;

int right_percent = 50;

int percentToMicroseconds(int p) {

if (p < 0) p = 0;

if (p > 100) p = 100;

// map 0..100 -> 1400..1800

return 1400 + p * 2;

}

void setLeftPercent(int p) {

left_percent = constrain(p, 0, 100);

left.writeMicroseconds(percentToMicroseconds(left_percent));

}

void setRightPercent(int p) {

right_percent = constrain(p, 0, 100);

right.writeMicroseconds(percentToMicroseconds(right_percent));

}

void handleSingleChar(char c) {

switch (c) {

case 'a':

// вращение влево (пример)

right.writeMicroseconds(1800);

left.writeMicroseconds(1200);

break;

case 'd':

right.writeMicroseconds(1200);

left.writeMicroseconds(1800);

break;

case 'x':

right.writeMicroseconds(1500);

left.writeMicroseconds(1500);

myservo.writeMicroseconds(1500);

break;

case 'w':

right.writeMicroseconds(1200);

left.writeMicroseconds(1200);

break;

case 's':

right.writeMicroseconds(1800);

left.writeMicroseconds(1800);

break;

case 'q':

right.writeMicroseconds(1800);

left.writeMicroseconds(1200);

break;

case 'e':

right.writeMicroseconds(1200);

left.writeMicroseconds(1800);

break;

case 'r':

myservo.writeMicroseconds(2000);

break;

case 't':

myservo.writeMicroseconds(1420);

break;

default:

// неизвестная команда

break;

}

}

void parseBuffer(String s) {

s.trim();

if (s.length() == 0) return;

// одиночный символ

if (s.length() == 1 && !isDigit(s.charAt(0))) {

handleSingleChar(s.charAt(0));

return;

}

// команда с префиксом + число, например L50, R75, P90

char cmd = s.charAt(0);

String valstr = s.substring(1);

int val = valstr.toInt(); // если не число — 0

switch (cmd) {

case 'L':

setLeftPercent(val);

break;

case 'R':

setRightPercent(val);

break;

case 'P':

// servo angle 0..180 -> use write(angle)

val = constrain(val, 0, 180);

myservo.write(val);

break;

default:

// неизвестная многосимвольная команда

break;

}

}

void setup() {

myservo.attach(5);

right.attach(2);

left.attach(3);

// Serial (USB) остаётся для отладки

Serial.begin(115200);

Serial.println("Mega: starting. Listening on Serial3 at 9600.");

// UART3 - pins 14(TX3),15(RX3) on Mega

Serial3.begin(9600);

// установить начальные скорости на 50%

setLeftPercent(50);

setRightPercent(50);

myservo.write(90); // нейтральный угол

}

void loop() {

while (Serial3.available() > 0) {

char c = Serial3.read();

// Принят символ. разбираем по newline, либо одиночный символ

if (c == '\n' || c == '\r') {

if (buf.length() > 0) {

parseBuffer(buf);

// debug

Serial.print("Parsed: ");

Serial.println(buf);

buf = "";

}

} else {

buf += c;

// если это одиночный нецифровой символ — обработать сразу

if (buf.length() == 1 && !isDigit(buf.charAt(0))) {

parseBuffer(buf);

Serial.print("Parsed single: ");

Serial.println(buf);

buf = "";

}

// защитный предел длины

if (buf.length() > 10) {

buf = ""; // сброс на случай ошибки

}

}

}

}